| Schedule 1153.07 Minimum Setback Requirements |  |  |
| --- | --- | --- |
|  | R-1 Single-Family Residential District | R-2 Two-Family Residential District |
| (1) Side Setback (a) |  |  |
| A. Minimum of any one side | 8 feet | 8 feet |
| B. Total side setback | 16 feet | 16 feet |
| (2) Rear Setback (b) | 25 ft. | 25 ft. |
| (a) For lots of less than sixty (60) feet in width, each side setback shall be not less than one-eighth (1/8) of the width of the lot, but not less than five (5) feet, whichever is greater. (b) The required rear setback, when the parcel is adjacent to Lake Erie, shall be determined by averaging the rear setbacks of the nearest dwellings on either side of the lot, but shall not be less than 25 feet. For the purposes of this Section only, the "property line" along Lake Erie shall be the point where the natural shoreline intersects the ordinary high water elevation as determined by the Ohio Department of Natural Resources, Division of Geological Survey. |  |  |

1153.09 HEIGHT REQUIREMENTS.

All buildings and structures in any residential district shall comply with the following height regulations:

(a)   The height of principal buildings and structures shall not exceed twenty-five (25) feet.

(b)   The height of accessory buildings and structures shall not exceed ten (10) feet, except the height of detached garages shall not exceed twelve (12) feet at the highest point of the structure for a flat roof, and shall not exceed fifteen (15) feet at the highest point of the structure for any other roof that is not a flat roof, notwithstanding the height definition defined in Section 1123.02(b)(19) of this Code.

(c)   Permitted height exceptions are set forth in Section 1181.01, Supplementary Height Regulations. (Ord. 87-17. Passed 12-17-18.)

1153.15 ACCESSORY USES AND STRUCTURES.

Accessory uses, buildings, and structures permitted in residential districts shall conform to the location, coverage, area, and maintenance standards contained in this Section and Chapter 1181, General Use Regulations. No accessory building or use shall be established on a lot unless a principal building or use has first been established on the lot in

(c)   Accessory Buildings. An accessory storage building shall not exceed one hundred (120) square feet in gross floor area. No more than two (2) accessory buildings, only one (1) of which shall be an accessory storage building, shall be located on a single zoning lot. The maximum, gross floor area of all accessory buildings on a zoning lot, including detached garages, shall not exceed 600 square feet. Accessory building shall not contain habitable spaces.

(h)    Private Swimming Pools. A private swimming pool, not including farm ponds; retention basins; and lakes, shall be any pool or open tank not located within a completely enclosed building and containing or normally capable of containing water to a depth at any point greater than one and one-half (1.5) feet. No such swimming pool, exclusive of portable swimming pools with a diameter less than twelve (12) feet or with an area of less than 100 square feet, shall be allowed in any R-1 or R-2 District, except as an accessory use and unless it complies with the following conditions and requirements:

(1)    Private swimming pools shall be located on the same zoning lot with the principle use to which it is an accessory use.

(2)    The pool shall not be located, including any walks or paved areas or accessory structure adjacent thereto, closer than eight (8) feet to any property line of the zoning lot on which it is located.

(3)    Every outdoor pool shall be enclosed by a fence forty- eight (48) inches in height around the perimeter of the pool, except the freestanding aboveground pools with sides greater than thirty-six (36) inches high from the ground on which its rests shall not require additional fencing, insofar as the side of such pool itself constitutes a barrier equal to a fence, and ladders and/or any means of access used to enter such pool shall be removed when not in use. One (1) or more gates shall be provided, and each is to be secured so as to prevent access by children. The location of the fence shall be at the discretion of the applicant, but must be at the approval of the Building Division. The fence shall be constructed of permanent and durable materials, and shall be fifty-percent (50%) or more open to light and air, which shall permit a person to see through into the swimming pool.