| Schedule 1153.07 Minimum Setback Requirements |  |  |
| --- | --- | --- |
|  | R-1 Single-Family Residential District | R-2 Two-Family Residential District |
| (1) Side Setback (a) |  |  |
| A. Minimum of any one side | 8 feet | 8 feet |
| B. Total side setback | 16 feet | 16 feet |
| (2) Rear Setback (b) | 25 ft. | 25 ft. |
| (a) For lots of less than sixty (60) feet in width, each side setback shall be not less than one-eighth (1/8) of the width of the lot, but not less than five (5) feet, whichever is greater. (b) The required rear setback, when the parcel is adjacent to Lake Erie, shall be determined by averaging the rear setbacks of the nearest dwellings on either side of the lot, but shall not be less than 25 feet. For the purposes of this Section only, the "property line" along Lake Erie shall be the point where the natural shoreline intersects the ordinary high water elevation as determined by the Ohio Department of Natural Resources, Division of Geological Survey. |  |  |