1153.15 ACCESSORY USES AND STRUCTURES.

Accessory uses, buildings, and structures permitted in residential districts shall conform to the location, coverage, area, and maintenance standards contained in this Section and Chapter 1181, General Use Regulations. No accessory building or use shall be established on a lot unless a principal building or use has first been established on the lot in conformance with all applicable provisions of this Development Code.

(a)    Location Requirements for Accessory Uses. An accessory building or use permitted in a residential district shall be located as set forth in Schedule 1153.15. However, an accessory use shall only be permitted to the extent such use complies with all other accessory use regulations set forth in this Development Code.

(b)    Schedule 1153.15 Permitted Accessory Structures In Front, Side And Rear Yards:

| Schedule 1153.15 Permitted Accessory Structures in Front, Side and Rear Yards |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Use | Yard Permitted | Minimum Setback From Lot Line |  |  |  |
|  |  | Front | Side | Rear |  |
| (1) Detached accessory buildings, including garages | Rear | NP | 5 ft. | 5 ft. |  |
| (2) Driveways | Front, corner side, side, rear | NA | 3 ft. | 3 ft. |  |
| (3) ATTACHED ARCHITECTURAL FEATURES | Rear | See also Section 1153.13 |  |  |  |
| (4) Fences, walls | Front, corner side, side, rear | 0 ft. | 0 ft. | 0 ft. |  |
| (5) Outdoor storage of recreation vehicle/equipment | Rear | See also Section 1153.15(m) |  |  |  |
| (6) Private Swimming Pools | Rear | NP | See also Section 1153.15(h) |  |  |
| (7) Play Structure, Pergola, Gazebo, PATIO FIREPLACE | Rear | NP | See also Section 1153.15(g) |  |  |
| (8) Boat House | Rear | NP | See also Section 1153.15(i) |  |  |

1153.09 HEIGHT REQUIREMENTS.
   All buildings and structures in any residential district shall comply with the following height regulations:

(a)   The height of principal buildings and structures shall not exceed twenty-five (25) feet.

(b)   The height of accessory buildings and structures shall not exceed ten (10) feet, except the height of detached garages shall not exceed twelve (12) feet at the highest point of the structure for a flat roof, and shall not exceed fifteen (15) feet at the highest point of the structure for any other roof that is not a flat roof, notwithstanding the height definition defined in Section 1123.02(b)(19) of this Code.

(c)   Permitted height exceptions are set forth in Section 1181.01, Supplementary Height Regulations. (Ord. 87-17. Passed 12-17-18.)