1153.15 ACCESSORY USES AND STRUCTURES.

Accessory uses, buildings, and structures permitted in residential districts shall conform to the location, coverage, area, and maintenance standards contained in this Section and Chapter 1181, General Use Regulations. No accessory building or use shall be established on a lot unless a principal building or use has first been established on the lot in conformance with all applicable provisions of this Development Code.

(c)   Accessory Buildings. An accessory storage building shall not exceed one hundred (120) square feet in gross floor area. No more than two (2) accessory buildings, only one (1) of which shall be an accessory storage building, shall be located on a single zoning lot. The maximum, gross floor area of all accessory buildings on a zoning lot, including detached garages, shall not exceed 600 square feet. Accessory building shall not contain habitable spaces.

(l)   Additional Regulations for Vehicles.

(4)   Accessory off-street parking spaces located in the front yard shall adjoin the driveway between the driveway and the nearest side lot line, provided parking spaces shall not be located directly in front of the dwelling.

A.   Parking areas, including driveways, for single family detached and two family dwellings shall be located not less than three (3) feet from adjoining property lines as provided in Section 1111.09(c)

B.   An accessory parking space meeting the standards above shall be no wider than eight (8) feet and no longer than eighteen (18) feet long with a return to the driveway within thirty (30) feet measured from the front of the garage. Accessory parking spaces are prohibited in cases of the setback being thirty five (35) feet or less or where a three (3) car garage is proposed or exists. Refer to drawing 1153.15(l)(4)D. for clarity.

C.   Accessory parking spaces in R-1 & R-2 districts will be screened between the property line and parking spaces with evergreen plant material in a manner that so that within four (4) years the vehicle is completely obscured from view.

D.   Driveways shall not be less than eight (8) feet or greater than twenty (20) feet in width in R-1 and R-2 Districts except where a three (3)-car garage exists or is proposed the driveway shall not exceed thirty (32) feet in width. Where the building setback is 35' or less a full width driveway is permitted in order to service two (2) and three (3) car garages. In R-1 and R-2 Districts, only one curb cut shall be permitted for any lot, except that two curb cuts shall be permitted for any lot that meets all of the following conditions:

1.   The lot width is ninety (90) feet or wider; and,

2.   The front setback for the lot is at least fifty (50) feet or greater; and,

3.   The proposed turn-around driveway and associated durable edging shall be no greater than twelve (12) feet in total width and,

4.   For corner lots, both curb cuts are to be located on the same street.